Excavation Safety

OVERVIEW

Purpose	1

Leadership and Accountability	1

Competent Persons	1

Employee Information and Training	1

Protective Systems	1

Outside Contractors	2

Soils Analysis Checklist	3

Daily Inspection Log	5

Trench Checklist	6

# Excavation Safety

# Purpose

- This policy was created to protect our employees from cave-in and trench collapse while they are working in or near trenches. This policy covers the correct procedures and equipment employees are to use while setting up and working in trenches.

- Each employee in an excavation shall be protected from cave-ins by an adequate protective system except when trenches are made entirely in stable rock or are less than 5 feet (1.52m) in depth and examination of the ground by a competent person provides no indication of a potential cave-in.

- Copies of this policy will be accessible to employees at location. Each employee working in or near a trench will be trained on this policy.

Leadership and Accountability

Name or Position will be responsible for annually evaluating the confined spaces entry policy effectiveness and selecting appropriate personal protection equipment and general entry equipment.

Employees are accountable to know and follow these policies. Employees are also accountable for the proper care, maintenance, and correct use of equipment provided for the job.

Employees should refer questions or comments about this policy to Name or Position.

Competent Persons

- The person designated as the competent person on a trench site shall have the authority to stop all work to correct or eliminate dangerous conditions.

- The competent person shall be on-site during all trenching activity.

- The competent person shall be responsible for completing a daily inspection log of the trench site.

Only employees receiving competent person training will be designated as the competent person for a specific trench. Copies of Competent Person Training for each employee will be included as an appendix to this program.

# Employee Information and Training
Training will be provided to all employees so they acquire the understanding, knowledge, and skills necessary for the safe performance of their duties. Training will be documented and the records stored at location.

  - See IMWCA Online Training Course

# Protective Systems

The person that is designated as the competent person for an trench site shall determine the appropriate protective system for that trench site and oversee the installation and conduct an inspection of the system before employees are allowed to work in the trench.

- For all trenches 20 feet or deeper, a registered engineer shall design the protective system.

# Outside Contractors

Whenever outside servicing personnel are to be engaged in activities covered by the scope and application of this standard, the on-site employer and the outside employer shall inform each other of their respective trench safety program and procedures.

All outside contractors shall be pre-qualified before the commencement of work on behalf of Member’s name.

Coordinator following job completion.

- Soils Analysis Checklist

This checklist must be completed when soil analysis is made to determine the soil type(s) present in the trench. A separate analysis must be performed if the trench (trench) is stretched over a distance where soil type changes.

| SITE LOCATION: |  |  |  |  |
| --- | --- | --- | --- | --- |
| DATE: | TIME: |  | COMPETENT PERSON: |  |
| WHERE WAS THE SAMPLE TAKEN FROM: |  |  |  |  |
| TRENCH DEPTH: |  | TRENCH WIDTH: |  | TRENCH LENGTH: |

VISUAL TEST

Particle type: Fine grained (cohesive) ________ Granular (sand/silt or gravel) _______

Water conditions: Wet ______ Dry ______ Seeping Water ______ Surface water present ______ Submerged ______

Previously disturbed soils: Yes ________ No ________

Underground utilities: Yes ______ No ______ If yes, what type?

Layered soils? Note: The least stable layer controls soil type. Yes ______ No ______

Layered soils dipping into trench: Yes ______ No ______ Unknown ______

Trench exposed to vibrations: Yes ______ No ______ If yes, from what?

Crack like openings or spalling observed: Yes ______ No ______

Conditions that may create a hazardous atmosphere: Yes ______ No ______ If yes, identify condition and source:

Surface encumbrances: Yes ______ No ______ If yes, what type?

Work to be performed near public vehicular traffic: Yes ______ No ______

Possible confined space exposure: Yes ______ No ______

Soils Analysis Checklist (continued)

- MANUAL TEST

Plasticity: Cohesive ______ Noncohesive ______

Dry strength: Granular (crumble easily) ______ Cohesive (broken with difficulty) ______

Wet shake: Water comes to surface (granular material) ____ Surface remains dry (clay material) ____

NOTE: The following unconfined compressive strength tests should be performed on undisturbed soils.

Thumb Test used to estimate unconfined compressive strength of cohesive soil:

Test performed: Yes ______ No ______

________ Type A – soil indented by thumb with very great effort.

________ Type B – soil indented by thumb with some effort.

________ Type C – soil easily penetrated several inches by thumb with little or no effort. If soil is submerged, seeping water, subjected to surface water, runoff, exposed to wetting.

Penetrometer or Shearvane used to estimate unconfined compressive strength of cohesive soils:

Test performed: Yes ______ No ______ Device used:

________ Type A – soil with unconfined compressive strength of 1.5 tsf or greater.

________ Type B – soil with unconfined compressive strength greater than 0.5 tsf and less than 1.5 tsf.

________ Type C – soil with unconfined compressive strength of 0.5 tsf or less. If soil is submerged, seeping water, subjected to surface water, runoff, exposed to wetting.

NOTE: Type A – no soil is type A if soil is fissured, subject to vibration, previously disturbed, layered dipping into trench on a slope of 4H:1V.

- SOIL CLASSIFICATION

Stable Rock ______ Type A ______ Type B ______ Type C ______

SELECTION OF PROTECTIVE SYSTEM (Appendix F)

Protective System: ______ Sloping (Appendix B) Specify angle ______

______ Timber shorting (Appendix C)

______ Aluminum hydraulic shoring (Appendix D)

______ Trench shield Maximum depth in this soil ______

- Daily Inspection Log

| DATE: | SIGNATURE: |
| --- | --- |
| WEATHER: | PROJECT: |

Was One Call System contacted: Yes ______ No ______

Protective system: Trench shield (box) ______ Wood shoring ______ Sloping ______ Other ______

Purpose of trenching: Drainage ______ Water ______ Sewer ______ Gas ______ Other ___________________________

Were visual soil test made: Yes ______ No ______ If yes, what type?

Were manual soil tests made: Yes ______ No ______ If yes, what type?

Type of soil: Stable Rock ______ Type A ______ Type B ______ Type C ______

Surface encumbrances: Yes ______ No ______ If yes, what type?

Water conditions: Wet ______ Dry ______ Submerged ______

Hazardous atmosphere exists: Yes ______ No ______ (If yes, follow confined space entry procedures, complete Confined Space Entry Permit, monitor for toxic gas)

Is trenching or trench exposed to public vehicular traffic (exhaust emission): Yes ______ No ______ (If yes, refer to confined space entry procedures, complete Confined Space Entry Permit, monitor for toxic gas)

Measurements of trench: Depth ______ Length ______ Width ______

Is ladder within 25 feet of all workers: Yes ______ No ______

Is excavated material stored two feet or more from edge of trench: Yes ______ No ______

Are employees exposed to public vehicular traffic: Yes ______ No ______ (If yes, warning vests required)

Are other utilities protected: (Water, sewer, gas, or other structures) Yes ______ No ______

Are sewer or natural gas lines exposed: Yes ______ No ______ (If yes, refer to confined space entry procedures policy, complete Confined Space Entry permit, monitor for toxic gas)

Periodic inspection: Yes ______ No ______ Last (date) ______________

Have all employees on-site received training in excavating: Yes ______ No ______

- Trench Checklist

(To be completed by a “Competent Person”)

| SITE LOCATION: |  |  |  |  |
| --- | --- | --- | --- | --- |
| DATE: | TIME: |  | COMPETENT PERSON: |  |
| SOIL TYPE (See attached form:) |  |  |  |  |
| SOIL CLASSIFICATION: |  | TRENCH DEPTH: |  | TRENCH WIDTH: |
| TYPE OF PROTECTIVE SYSTEM USED: |  |  |  |  |

(Indicate for each item:   YES  --   NO  --   or N/A for not applicable)

| 1.) General inspection of job site: |  |
| --- | --- |
| - Trenchs, adjacent areas, and protective systems inspected by a competent person daily Prior to the start of work. |  |
| - Competent person has the authority to remove employees from the trench immediately. |  |
| - Surface encumbrances removed or supported. |  |
| - Employees protected from loose rock or soil that could pose a hazard by falling or rolling into the trench. |  |
| - Hard hats worn by all employees. |  |
| - Spoils, materials, and equipment set back at least two feet from the edge of the trench. |  |
| - Barriers provided at all remotely located trenchs, wells, pits, shafts, etc. |  |
| - Walkways and bridges over trenchs four feet or more in depth are equipped with standard guardrails and toeboards. |  |
| - Warning vests or other highly visible clothing provided and worn by all employees exposed to public vehicular traffic. |  |
| - Employees required to stand away from vehicles being loaded or unloaded. |  |
| - Warning system established and utilized when mobile equipment is operating near the edge of the trench. |  |
| - Employees prohibited from going under suspended loads. |  |
| - Employees prohibited from working on the faces of sloped or benched trenchs above other employees. |  |
| 2.) Utilities: |  |
| A. Utility companies contacted and/or utilities located. |  |
| B. Exact location of utilities marked. |  |
| C. Underground installations protected, supported, or removed when trench is open. |  |

- Trench Checklist (continued)

| 3.) Means of Access and Egress: |  |
| --- | --- |
| - Lateral travel to means of egress no greater than 25 feet in trenchs four feet or more in depth. |  |
| - Ladders used in trenchs secured and extended three feet above the edge of the trench. |  |
| - Structural ramps used by employees designed by a competent person. |  |
| - Structural ramps used for equipment designed by a registered professional engineer. |  |
| - Ramps constructed of materials of uniform thickness, cleated together on the bottom, equipped with no-slip surface. |  |
| - Employees protected from cave-ins when entering or exiting the trench. |  |
| 4.) Wet Conditions: |  |
| A. Precautions taken to protect employees from the accumulation of water. |  |
| B. Water removal equipment monitored by a competent person. |  |
| C. Surface water or runoff diverted or controlled to prevent accumulation in the trench. |  |
| D. Inspections made after every rainstorm or other hazard increasing occurrence. |  |
| 5.) Hazardous Atmosphere: |  |
| - Atmosphere within the trench tested where there is a reasonable possibility of an oxygen deficiency, combustible, or other harmful contaminant exposing employees to a hazard. |  |
| - Adequate precautions taken to protect employees from exposure to an atmosphere containing less than 19.5 percent oxygen and/or to other hazardous atmosphere. |  |
| - Ventilation provided to prevent employee exposure to an atmosphere containing flammable gas in excess of 10 percent of the lower explosive limit of the gas. |  |
| - Testing conducted often to ensure that the atmosphere remains safe. |  |
| - Emergency equipment, such as breathing apparatus, safety harness and lifeline, and/or basket stretcher readily available where hazardous atmospheres could or actually do exist. |  |
| - Employees trained to use personal protective and other rescue equipment. |  |
| - Safety harness and lifeline used and individually attended when entering bell bottom or other deep confined trenchs. |  |
| 6.) Support Systems: |  |
| - Materials and/or equipment for support systems selected based on all analysis, trench depth, and expected loads. |  |
| - Materials and equipment used for protective systems inspected and in good condition. |  |
| - Materials and equipment not in good condition have been removed from service. |  |
| - Damaged materials and equipment used for protective systems inspected by a registered professional engineer after repairs and before being placed back into service. |  |
| - Protective systems installed without exposing employees to the hazards of cave-ins, collapses, or threat of being struck by materials or equipment. |  |
| - Members of support system securely fastened to prevent failure. |  |
| - Support systems provided to ensure stability of adjacent structures, buildings, roadways, sidewalks, walls, etc. |  |

- Trench Checklist (continued)

| H. Trenchs below the level of the base or footing supported, approved by an RPE. |  |
| --- | --- |
| I. Removal of support systems progresses from the bottom and members are released slowly as to note any indication of possible failure. |  |
| J. Backfilling progresses with removal of support system. |  |
| K. Trench of material to a level no greater than two feet below the bottom of the support system and only if the system is designed to support the loads calculated for the full depth. |  |
| L. Shield system placed to prevent lateral movement. |  |
| M. Employees are prohibited from remaining in shield system during vertical movement |  |