MODEL PROGRAM:

Lockout/Tagout Program

Purpose	1

Leadership and Accountability	1

Key Definitions	1

Equipment and Machinery Evaluations	2

Cord and Plug equipment	2

Lockout/Tagout Procedures	2

Restoring Power	2

Group lockout/tagout	3

Shift Change	3

Isolation Devices	3

Emergency Lock/Tag Removal	3

Outside Contractors	3

Lockout/Tagout Program

# Purpose

- This policy was created to protect our employees from amputations and electrical burns while they service and repair equipment. This policy covers the control of hazardous energy sources and the procedures to protect our employees from exposure while at work.

Copies of this policy will be accessible to employees at location. Each employee repairing or servicing equipment will be trained on this policy.

# Leadership and Accountability

Name or Position will be responsible for annually evaluating the lockout/tagout policy effectiveness and selecting appropriate equipment.

Employees are accountable to know and follow these policies. Employees are also accountable for the proper care, maintenance, and correct use of equipment provided for the job.

Employees should refer questions or comments about this policy to Name or Position.

# Definitions
Affected employee. An employee whose job requires him/her to operate or use a machine or equipment on which servicing or maintenance is being performed under lockout or tagout, or whose job requires him/her to work in an area in which such servicing or maintenance is being performed.

Authorized employee. A person who locks out or tags out machines or equipment in order to perform servicing or maintenance on that machine or equipment. An affected employee becomes an authorized employee when that employee's duties include performing servicing or maintenance covered under this section.

Energy source. Any source of electrical, mechanical, hydraulic, pneumatic, chemical, thermal, or other energy.

Lockout. The placement of a lockout device on an energy-isolating device, in accordance with an established procedure, ensures that the energy-isolating device and the equipment being controlled cannot be operated until the lockout device is removed.

Tagout. The placement of a tagout device on an energy isolating device, in accordance with an established procedure, indicates that the energy isolating device and the equipment being controlled may not be operated until the tagout device is removed.

# Equipment and Machinery Evaluations

All equipment and machinery operated or serviced by employees shall be individually evaluated for an equipment or machine-specific lockout procedure using the Equipment and Machinery Evaluation form. The completed form shall be maintained in a binder or affixed to the equipment. Forms shall be reviewed and updated when equipment is changed or added.

# Download: Equipment and Machinery Evaluation Form

Download: Lockout/Tagout Equipment Assignments Form

# Cord and Plug Equipment

Equipment that can be unplugged from an outlet and has no other hazardous energy sources does not fall under the requirements of this program as long as the plug is under the exclusive control of the employee performing service or maintenance. If the employee is unable to maintain exclusive control of the plug, then a plug lock or other suitable energy isolation device must be used in accordance with this policy.

# Lockout/Tagout Procedures

- Prior to initiating the lockout/tagout procedure, all employees servicing or providing maintenance on machinery or equipment shall review the Equipment and Machinery Evaluation form to identify which energy sources are present and the procedures to follow to isolate the energy sources.

- Notify all affected employees that the equipment or machinery will be locked out or tagged out for servicing or repair. Either through a team meeting atmosphere or an announcement of some kind.

- If equipment is operating, shut down by normal means.

- Operate switches, valves, or other energy isolation devices so that equipment is effectively isolated from all energy sources. All stored energy must be dissipated, bled, or blocked to prevent accidental release.

- Apply appropriate isolation devices with assigned locks or tags. Tags may only be used if a machine cannot be locked. A tag will be attached to each lock to identify the installer, date, time and authorization of the lockout. If machinery or equipment cannot be locked out, a tag will be used by itself. If machinery or equipment can be locked out, then a lock must be used. 
Tags must be of durable construction and labeled in a manner that will remain legible in wet or corrosive environments. Tags will be attached by self-locking nylon cable when possible.

- Test all switches and operating controls to confirm that all energy sources are isolated. After test, return switches and other controls to “off” or “neutral.”

- Begin service or maintenance work.

Restoring Power

- After service or maintenance work is complete, inspect area to ensure tools are removed, re-install guards and verify that employees are clear of the machinery or equipment.

- Notify employees that lockout/tagout has concluded and the machine will be restarted.

- Have the same employees that originally placed the locking devices remove locks or tags.

- Commence start-up.

# Group lockout/tagout

If more than one person is required to lockout/tagout equipment or machinery, each person must place their personal lockout device or tag on the energy isolation device. If the equipment or machine will not accept multiple locks, a hasp may be used.

# Shift Change

The policy leader or the employee originating the lockout/tagout procedure for the specific job must be responsible for the transfer of lockout/tagout devices between shift personnel. All personnel MUST be informed of what is locked out and when it will be repaired

# Isolation Devices
An inventory of isolation devices and where they are located is included in the Lockout/tagout assignment form. Employee lock assignments are also included in this form. Locks issued for lockout may only be used for lockout procedures and cannot be used for any other use.

# Emergency Lock/Tag Removal

Locks or tags will only be removed in cases where the authorized employee who applied the lock or tag is unavailable. All emergency removal shall be done by a supervisor.

Employee Information and Training
The employer shall provide training so that all employees acquire the understanding, knowledge, and skills necessary to safely perform their duties. All training must be documented.

  - See IMWCA Online training

# Outside Contractors

Whenever outside servicing personnel are to be engaged in activities covered by the scope and application of this standard, the on-site employer and the outside employer shall inform each other of their respective lockout or tagout procedures.