Personal Protective Equipment (PPE)

OVERVIEW

PURPOSE	2

PROGRAM ADMINISTRATOR	2

EMPLOYER/EMPLOYEE RESPONSIBILITIES	2

REQUIRED PROGRAMS – BASED ON EMPLOYER ASSESSMENT OF WORKPLACE HAZARDS	3

OTHER RELATED ISSUES	3

OSHA PPE GENERAL REQUIREMENTS	4

PERSONAL PROTECTIVE EQUIPMENT NEEDS ASSESSMENT	5

Personal Protective Equipment (PPE)

## Purpose

This program was created to ensure the safety and health of employees when using Personal Protective Equipment (PPE). Personal protective equipment is designed to protect employees from potential workplace hazards. This program will enable the entity to comply with OSHA regulation Section 1910.Subpart I Personal Protective Equipment.

Copies of this program will be available to all employees in the workplace.

## Program Administrator

The program administrator is responsible for the hazard assessment required for this program. Upon completion of the assessment, the program administrator will select and purchase the needed equipment. The administrator will then ensure employees are trained on the proper fit, usage and maintenance of the required PPE. The program administrator is also charged with an annual review of the program.

Employees should refer their questions or comments about this program to the administrator. The program administrator is Name/Title.

## Employer/Employee Responsibilities

Employees shall be trained in PPE usage guidelines, including:

- what PPE is necessary for each situation encountered in the workplace,

- how to properly adjust, put on, wear and remove PPE,

- limitations of the specific PPE provided by the employer, and

- proper maintenance, useful life and disposal of PPE.

The employer must retrain if previous training becomes obsolete, there are substantial workplace changes, or if new PPE is introduced.

The employee is responsible for the maintenance and storage of PPE as required by specific training.

The selection of the proper PPE shall be made by the employer after a hazard assessment of the job is made. Non-mandatory Compliance Guidelines for Hazard Assessment and Personal Protective Equipment Selection may be found at:

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910SubpartIAppB

## Required Programs – Based on Employer Assessment of Workplace Hazards

#### Eye and Face Protection

Suitable eye protectors (safety glasses, goggles, face shields, wire mesh masks, etc.) must be provided where there is potential for injury to the eyes or face from flying particles, molten metal, liquid chemicals, acids or caustic liquids, chemical gases or vapors, body fluids, potentially injurious light radiation or a combination of these. To read the standard go to: https://www.osha.gov/pls/oshaweb/owadisp.show_document?p_table=STANDARDS&p_id=9778

#### Respiratory Protection

Devices such as dust masks, canister respirators, self-contained breathing apparatus or other such apparatus must be provided to employees who are exposed to harmful dust, fogs, fumes, mists, gases, smoke, sprays, or vapors. Persons working in oxygen-deficient or oxygen-enhanced atmospheres must also be protected. Persons using the devices must be fit-tested to the device, tested to see if they are physically able to use the devices, and trained in the use and care of the devices. To read the standard go to: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.134

#### Head Protection

Employees working in a location where there is danger of being struck in the head by falling objects or other dangers from above such as electrical hazards must wear head protection. To read the standard go to: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.135

#### Foot Protection

Persons must have foot and leg protection if their feet are subject to crushing foot injury. The feet must also be protected from puncture by sharp objects, molten metal, hot surfaces, wet and slippery surfaces. This also requires leg protection for persons using chainsaws and other devices that could cause injury to legs. To read the standard for to:

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.136

#### Electrical Protective Equipment

Persons involved in power generating or power distribution construction and maintenance must be protected from shock, electrocution and burn hazards. Appropriate clothing and PPE will be provided based on exposure Gloves, sleeves and fiberglass tools will be inspected by a qualified person. To read the standard go to: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.137

#### Hand Protection

Worker's hands and arms must be protected from cuts, burns, chemicals, bodily fluids and other recognized hazards. Proper hand wear must be selected for the task. Material safety data sheets specify the type of hand wear needed for handling various chemicals. To read the standard go to: https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.138

#### Occupational Noise Exposure – Hearing Conservation

The noise level of the workplace must be measured to determine if hearing protection is required. Hearing protection must be provided for persons exposed to noise levels exceeding the limits outlined in 1910.95. Employees must be tested for hearing capacity to establish a baseline for use when succeeding tests are made. The employer must furnish ear protection that will diminish the noise to acceptable

levels. Foam earplugs, ear “muffs”, or other suitable methods may be used to reduce noise exposure when no other way to reduce the noise level is possible. Employees’ hearing must be tested annually and compared to the baseline test to see if the hearing conservation program is effective. If the tests reveal further hearing loss, the employer must make every effort to further reduce the noise level exposure to the employee. To read the standard go to:

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.95

## Other Related Issues

#### Water Hazards

A U.S. Coast Guard approved life jacket or buoyant work vest is required when there is danger of falling into water at the work site. For emergency rescue operations, boats and ring buoys with at least 90 feet of line are required when working over or on water.

#### Visibility issues

Class 2 Hi Visibility clothing is required when working in or near the road right-of-way and exposed to traffic hazards. Class 3 Hi Visibility clothing with reflective material designed for high nighttime visibility is recommended for nighttime or low light conditions.

#### Seatbelts

Seatbelt use is mandatory when driving a motor vehicle. Employees must use seatbelts when machines or equipment are factory equipped with the devices.

#### Personal wear items

The employer must furnish all PPE, except personal wear items. Personal wear items worn frequently or daily, such as safety toed footwear or prescription safety glasses are usually purchased by the employee, though the employer may choose to share the cost. Specialized footwear such as steel toed hip boots or other items used infrequently should be furnished by the employer.

Personal wear items should be replaced on a timely basis, e.g., yearly, every second year or whenever the item is damaged as a result of work activity. For example, an employee inspecting on an asphalt- paving project has safety toe shoes ruined by the tack oil and hot mix. This would be abnormal wear and the cost should be borne by the employer. These situations should be judged on a case-by-case basis.

## OSHA PPE General Requirements

View and download Section 1910.132 – 138 Personal Protective Equipment Standards from the OSHA website, click on the following link:

https://www.osha.gov/laws-regs/regulations/standardnumber/1910/1910.132

Print out a copy of the standard and mandatory appendices to include with your program.

# Personal Protective Equipment Needs Assessment

Sample

Department/Division:

### Check each of the types of personal protective equipment that would be required for the listed task.

| Tasks or work functions that are performed by employees of the department or division: | Safety Glasses | Hard Hat | Traffic Safety Vest | Hearing Protection | - Muffs | - Plugs | Gloves | - Leather | - Chemical Resistant | - Latex | Face Shield | Chemical Apron | Welding Goggles | Welding Face cover | Welding Gloves | Flame resistant clothes | Safety toed shoes | Di-electric safety shoes | Chainsaw Chaps | Chemical Goggles | Respiratory Protection | - Self Contained | - Air Purifying |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Brush cutting |  | x | x |  | x |  |  | x |  |  | x |  |  |  |  |  | x |  | x | x |  |  |  |
| Changing chlorine cylinders |  |  |  |  |  |  |  |  | x |  | x | x |  |  |  |  |  |  |  |  |  | x |  |