MODEL PROGRAM:

Crane and Hoist Safety

Purpose	1

Leadership and Accountability	1

List of Hoist and Location	1

Operating Procedures	1

Inspection Procedure	3

Employee Information and Training	3

Crane and Hoist Safety

# Purpose

This policy and set of procedures were created to protect our employees’ safety while operating or working with a hoist at work.

Copies of this policy and procedures will be accessible to employees at location.

# Leadership and Accountability
Name or Position will be responsible for annually evaluating the hoist use policy and procedures effectiveness and monitoring hoist use within the facilities.

Employees are accountable for the proper care, maintenance, and proper use of the hoist and other load-moving equipment.

Employees should refer questions or comments about this policy to Name or Position.

# List of Hoist and Location

| Hoist Description | Location |
| --- | --- |
| 2 TON CM LODESTAR 8 FPM 115V SINGLE PHASE | Mechanics bay – roads shop |

# Operating Procedures
The operator shall perform the following duties:

- A qualified operator or trainee shall operate equipment under the direct supervision of the qualified operator. Exception: Maintenance and test personnel and inspectors, when in the performance of their duties, shall be allowed access only after permission has been granted by the operator.

- When operating the equipment, the operator shall maintain full attention on the task being performed, e.g., no use of headsets, or music.

- The operator shall ensure that hand signals used during the lift are understood and followed by all involved.

- No load in excess of the rated capacity shall be lifted unless for test purposes, and the test shall be an engineered lift.

- Before leaving the hoist unattended, the operator shall land any load, place the controls or master switch in the off position, and open the specific hoist’s main line device.

- The main line disconnect shall not be closed until the operator has ensured no one is on or adjacent to the hoist.

- If the hoist has been locked out or tagged out, the operator shall not remove the lock or tag unless the lock or tag has been placed there by the operator. To remove someone else's lock or tag, refer to the Lockout/Tagout program in the member's Safety Manual.

- For cab-operated equipment, ensure that all controls are in the "off" position prior to closing the line disconnect.

- During the use of cab-operated equipment, if the power should fail, the operator shall turn off all controllers. Before restarting, the operator shall check the motion controls for proper direction to ensure controls are in the neutral position.

- Persons boarding or leaving cab-operated equipment shall do so at the designated point of access or egress.

Attaching the Load

- Hoist chains or ropes shall be free of kinks or twists.

- Hoist chains or ropes shall not be wrapped around the load.

- The load shall be attached to the load block.

- Before lifting the load, the operator shall ensure that the load, sling, attachments, lifting devices, and the load block are unobstructed.

Moving the Load

- The person responsible for directing the lift shall make sure that the load is properly secured, balanced, and positioned in the sling or other lifting device.

- The person responsible for directing the lift shall make another visual inspection of the hoist chain or rope to ensure there are no kinks or twists.

- The load block shall be brought over the load in a manner that will prevent swinging when lifting the load.

- The chain or rope shall be inspected to ensure that it is properly seated in the chain sprocket or drum groove.

- Lift equipment shall not be used for side pulls.

- The operator shall not lift, travel, or lower a load while someone is on the load or hook.

- The operator shall avoid lifting the load over people.

- If the load being lifted approaches the rated load to be handled, the operator shall test the brakes by lifting the load a few inches and applying the brakes.

- The load shall not be lowered below the point where there are less than two wraps of rope on the hoisting drum unless a lower-limit device is provided. If a lower-limit device is provided, no less than one wrap shall remain.

Parking the Load

- The operator shall not leave a suspended load unattended.

- The load block of the hoist shall be raised above head level when not in use.

Hand Signals
Hand signals shall be used unless the participants of the lift are equipped with telephones, radios, or other equivalent means of communication.

# Inspection Procedure

Cranes in Periodic Use

A crane or overhead gantry that has not been used for one month or more will be inspected before each use and the focus for such an inspection is as follows:

- Inspect all functional operating mechanisms.

- Check for damage to or leaks from lines, tanks, valves, drain pumps, and air or hydraulic systems.

- Check the load hook for deformities or cracks.

- Check all hoist chains for excessive wear, including end connectors.

- Check all chains for kinks, twists and distorted links and stretches beyond what is recommended by the manufacturer.

- Inspect the rope for damage such as kinks, cracks, cutting, bending, broken wires, and unraveling, corroded, or improperly connected end connections.

Regardless of how often a crane or overhead gantry is used, the unit shall be inspected annually. The inspection shall be performed by an outside contractor qualified to inspect the unit and the contractor shall document and provide the owner with a copy of the findings.

Maintenance

Preventive maintenance shall be performed as prescribed by the manufacturer as detailed in the owner's manual. Maintenance of the units shall be performed by an outside contractor qualified to perform maintenance.

Adjustments and Repair

Any unsafe condition noted during the inspection of the crane shall be repaired before the crane is used.

# Employee Information and Training
Training will be provided to all employees, so they acquire the understanding, knowledge, and skills necessary to safely perform their duties. Training will be documented and the records stored at location.

  - See IMWCA Learn Online class

Curriculum

The curriculum of the training program shall, at a minimum, address the following topics:

- Wire rope

- Slings

- Occupational Safety and Health Administration (OSHA) and American National Standards Institute (ANSI) standards

- Hoisting equipment manual and power

- Operation and safety awareness, including lock and tag procedures

- Basic rigging

- Field training and trials

- Inspection procedures

Retraining

Employees shall be required to participate in annual refresher training. Retraining may also be deemed necessary when it has been documented that the operator has failed to operate the crane in a safe and appropriate manner as directed by this policy and according to OSHA and ANSI regulations.