MODEL PROGRAM:

Forklift Safety Program

Purpose	1

Leadership and Accountability	1

Operator Training	1

Certified Operators	2

Forklift Inventory	2

Powered Industrial Truck Rules for Safety	2

Model Inspection Form	6

Forklift Safety Program

# Purpose

This policy was created to protect our employees that operate or work around forklifts at work. This policy covers the safe operating and fueling procedures for the type of forklift we use. This program also serves to help the employer’s name and its employees comply with Occupational Safety and Health Administration (OSHA) Powered Industrial Trucks requirements as found in 29 CFR 1910.178.

Copies of this policy will be given to each employee that operates a forklift. Each employee exposed to forklifts as part of their job duties will be trained on this policy.

# Leadership and Accountability

Name or Position will be responsible for an annual review of the policy and ensuring that forklift operators are evaluated every three years. Name or Position will review forklift operator training, purchase of equipment and incident review of injuries or illness involving forklifts.

Employees are accountable for the proper care, maintenance, and proper use of the equipment.

Employees should refer questions or comments about this policy to Name or Position.

# Operator Training

Only employees who have successfully completed training in accordance with 1910.178(l) will be permitted to operate a powered industrial truck.

The employer shall certify that each operator has been trained and evaluated as required by this paragraph (l). The certification shall include the operator’s name, the date of the training, the date of the evaluation, and the identity of the person(s) performing the training or evaluation.

An evaluation of each operator's performance will be conducted at least once every three years.

If an operator has previously received training in a topic specified in paragraph 29 CFR 1910.178, and the training is appropriate to the truck and working conditions encountered, additional training in that topic is not required if the operator has been evaluated and found competent to operate the truck safely.

  - See IMWCA online Learn training.

# Certified Operators

| Name | Department or Location | Last Certification Date |
| --- | --- | --- |

# Forklift Inventory

| Manufacturer | Class | Fuel Type |
| --- | --- | --- |
| Nissan | Class I | Propane |
| Toyota | Class VII | Gas |

# Powered Industrial Truck Rules for Safety

The following is a list of safety rules pertaining to the operation of a powered industrial truck.

- Truck Operations:

- A safe distance will be maintained from the edge of ramps or platforms while on any elevated dock, platform or freight car.

- When leaving the truck unattended, the forks will be fully lowered, the controls placed in neutral, the power shut off, the brakes set to and the key or connector plug removed. The wheels will be blocked if the truck is parked on an incline. Note: A powered industrial truck is considered unattended when the operator is 25 feet or more away from the vehicle, which remains in his/her view, or whenever the operator leaves the vehicle and the truck is not in view.

- When the operator of an industrial truck is dismounted and within 25 feet of the truck still in his view, the load engaging means shall be fully lowered, controls neutralized, and the brakes set to prevent movement.

- Trucks will not be used to open or close freight doors.

- The brakes of trucks, trailers and railroad cars will be set and wheel chocks or stops will be in place to prevent movement during loading or unloading operations. Fixed jacks may be necessary to support a semi-trailer during loading or unloading when the trailer is not coupled to a tractor. The operator will check the flooring of trucks, trailers and railroad cars for breaks and problem areas before driving these vehicles into these surfaces.

- An overhead guard will be used as protection against falling objects. Note: The overhead guard is intended to offer protection from the impact of small packages, boxes, or bagged materials only.

- A load backrest extension will be used whenever necessary to minimize the possibility of the load or part of the load from falling backward.

- Fire doors, access to stairways, fire extinguishers and emergency exits will always be kept clear.

- Powered industrial trucks will not be driven up to anyone standing in front of a bench or other fixed object.

- No person will be allowed to stand or pass under the elevated portion of any truck, whether loaded or empty.

- Passengers are not permitted to ride on powered industrial trucks unless authorized and the truck is equipped with a safe place for the passenger to ride.

- The operator will never place his/her arms or legs between the uprights of the mast or outside the running lines of the truck.

- The operator will never push one load with another load.

- Spinner knobs must not be attached to the steering handwheels of trucks not originally equipped with such knobs.

- Never lift people on the forks of a powered industrial truck unless the truck has a properly designed safety platform securely attached to the lifting carriage and/or forks. If the truck is equipped with vertical controls only, or vertical and horizontal controls elevatable with the lifting carriage or forks, means will be provided whereby personnel on the platform can shut off power to the truck. Protection from falling objects as indicated necessary by the operating conditions will also be provided.

- Safety platforms, firmly secured to the lifting carriage and/or forks, shall be used.

- Traveling:

- Traffic regulations will be observed, including observing all stop signs and authorized plant speed limits.

- A safe distance of approximately three truck lengths from the truck ahead will be maintained whenever possible.

- The “Right of Way” will be yielded to ambulances or other vehicles in emergencies.

- The operator will slow down and sound the horn at intersections and other locations where vision is obstructed.

- If the load being carried obstructs the forward view, the operator will travel in reverse with the load trailing.

- Grades will be ascended or descended slowly. When ascending or descending grades in excess of 10 percent, loaded trucks will be driven with the load upgrade. Unloaded trucks will be operated on all grades with the load engaging means downgrade. On all grades, the load and load engaging means will be tilted back and raised only as far as necessary to clear the road surface.

- The operator will slow down for wet and slippery floors.

- Dock boards or bridge plates will be properly secured before they are driven over and their rated capacity will never be exceeded. They will always be driven over carefully and slowly.

- Elevators will be approached slowly and then entered squarely after the elevator car is properly leveled. Once on the elevator, the transmission will be in neutral, the engine shut off and the brakes set to prevent movement.

- When making turns, the operator will reduce the truck’s speed to a safe level by means of turning the hand steering wheel in a smooth, sweeping motion. Except when maneuvering at a very low speed, the hand steering wheel shall be turned at a moderate, even rate.

- Other trucks traveling in the same direction or at intersections, blind spots or other dangerous locations will not be passed.

- Horseplay and stunt driving, including spinning of the tires, is not permitted.

- Running over loose objects in aisle ways will be avoided.

- Under all travel conditions, the truck will be operated at a speed that will permit the truck to be brought to a stop in a safe manner.

- The operator will always look in the direction of travel and keep a clear view of the path of travel.

C.	Loading/Stacking:

- Only stable and safely arranged loads will be handled. Use extreme caution when handling off-centered loads that cannot be centered on the forks.

- Only loads within the rated capacity of the truck will be handled.

- The forks will be placed under the load as far as possible and the mast carefully tilted backward to stabilize the load.

- Extreme care will be used when tilting the load forward or backward especially when high tiering. Tilting forward with load engaging means elevated shall be prohibited except to pick up a load. An elevated load will not be tilted forward except when the load is in a deposit position over a rack or stack of material.

- When stacking or tiering loads, the operator will tilt the load backward only enough to stabilize the load.

- The operator will remove unsafe containers and pallets from service.

- Trucks equipped with attachments will be operated as partially loaded trucks when not handling a load.

- The operator will adjust long and high loads, including multiple-tiered loads that may affect the capacity of the truck.

- The operator will ensure there is always a safe distance between the mast and overhead lights, pipes and sprinkler systems.

D.	Maintenance of the Truck:

- Powered industrial trucks will be inspected before being placed in service. This inspection will be made at least daily. Trucks used on a round-the-clock basis will be inspected after each shift. *The results of these inspections will be documented on a Powered Industrial Truck Inspection Checklist (See Samples 1 and 2)

- If a truck is found to be unsafe at any time during the driver's shift, the operator will immediately notify his/her supervisor and remove the truck from service until it has been restored to safe operating condition.

- Fuel tanks shall not be filled while the engine is running. Spillage shall be avoided.

- The operator will always wear the proper personal protective equipment when fueling the truck or performing any other maintenance on the truck.

- No truck will be operated with a leak in the fuel system until the leak has been corrected.

- Open flames will not be used to check the electrolyte level in batteries or the fuel tank’s gasoline level.

- Smoking is not allowed while changing LPG tanks, refueling gas-powered trucks or changing or charging batteries for electric-powered vehicles.

# E.	Equipment Inspection and Maintenance- Daily

- When trucks are used on a round-the-clock basis, each truck will be inspected after each shift. *The results of these inspections will be documented on a Powered Industrial Truck Inspection Checklist.

- The operator will immediately notify his/her supervisor if the truck is found to be in need of repair and/or unsafe.

- If repairs are needed on a powered industrial truck that prevents its safe operation, the truck will be taken out of service until the repairs have been made.

- Repairs must be made by authorized personnel only.

- When the temperature of any part of any truck is found to be in excess of its normal operating temperature, the vehicle must be removed from service and not returned to service until the cause for the overheating has been eliminated.

- Any vehicle that emits hazardous sparks, flames or smoke from the exhaust system will be removed from service and not returned from service until the cause for the hazardous emissions has been corrected.

- Powered industrial trucks are to be kept in a clean condition and free of excess lint, oil and grease. Only non-combustible agents should be used for cleaning trucks. Cleaning trucks with low flash point solvents (below 100 degrees Fahrenheit) is not permitted.

- Precautions regarding toxicity, ventilation, personal protective equipment and fire hazards are to be followed as stated on the warning label and/or the Safety Data Sheet (SDS) for that particular cleaning agent.

- Parts used in any industrial truck requiring replacement will be replaced only with parts equal in safety to those parts originally provided by the manufacturer.

# Model Inspection Form

Truck Number: ____________________________

### Hour Meter Reading: _______________________

Date of Inspection: _________________________

Inspected By: ______________________________

| Check each item | Condition |  | Explain below if not OK |
| --- | --- | --- | --- |
|  | OK | Not OK |  |
| Key off procedures |  |  |  |
| Overhead guard |  |  |  |
| Hydraulic Cylinders |  |  |  |
| Mast assembly |  |  |  |
| Lift chains and rollers |  |  |  |
| Forks |  |  |  |
| Tires |  |  |  |
| LPG Tank and Locator pin |  |  |  |
| LPG tank hose |  |  |  |
| Gas gauge |  |  |  |
| Battery |  |  |  |
| Hydraulic Fluid level |  |  |  |
| Engine oil level |  |  |  |
| Engine coolant level |  |  |  |
| Check each item | Condition |  | Explain below if not OK |
|  | OK | Not OK |  |
| ## Key on procedures |  |  |  |
| Front, tail and brake lights |  |  |  |
| Oil pressure indicator lamp |  |  |  |
| Ammeter indicator lamp |  |  |  |
| Hour meter |  |  |  |
| Water temperature gauge |  |  |  |
| Steering |  |  |  |
| Brakes |  |  |  |
| Horn |  |  |  |
| Safety seat (if equipped) |  |  |  |
| Load handling attachments |  |  |  |
| Transmission fluid level |  |  |  |